2. Construction science principles

# Worksheet 2a: Properties of concrete

Concrete is a material widely used in construction. Provide an analysis of the its properties by completing the table below. Use the internet or other sources to help you in your research.

| Composition | Concrete is made up of sand (which is a fine aggregate),ballast (which is a coarse aggregate), cement (which can be referred to as a binder) and water (which is an additive). |
| --- | --- |
| Uses in construction | Many uses: pre-cast concrete, concrete masonry units, bridge and road infrastructure, housing, floor screed, etc. |
| Durability | Highly durable material which can last for many years, particularly if not exposed to extreme environments. |
| Compressive strength | Concrete has a relatively high compressive strength and does not crack easily under weight. The compressive strength is typically controlled by the ratio of water to cement when forming he concrete. |
| Tensile strength | Concrete has a low tensile strength, which can be increased using additives, typically steel, to create reinforced concrete. |
| Conductivity | Concrete has a much lower thermal conductivity than metals, but significantly higher than other building materials such as timber. It is a poor insulator. |
| Thermal expansion | Expands when hot or when moisture content changes. Expansion joints are required in concrete buildings, bridges and other structures. The joints will widen during cold weather because the concrete contracts. Joints get narrower during hot weather as the concrete expands. |
| Effects of environmental conditions | Can be damaged by fire, aggregate expansion, seawater effects, bacterial corrosion and other physical and chemical action. |
| Degradation or failure | Concrete can deteriorate for a variety of reasons, and damage is often the result of long-term exposure to harsh environmental conditions. Concrete that is subjected to long-duration forces may be prone to ‘creep’, where the concrete moves or is permanently deformed to relieve stresses in the material. |