2. Construction science principles

# Worksheet 3: Materials tests (learner)

In order for materials to meet their required standards, testing has to be undertaken to check that a supplied material complies with its specification.

At your tutor’s direction, undertake one or all of the following simple tests and record the results, following your tutor’s instructions.

- A silt test for a mortar sand using the following link and equipment supplied.

- https://theconstructor.org/building/building-material/test-silt-content-sand/34520/

Record your results in the table. If you have time to carry out more than one test on the sand, you will obtain a more accurate result based on average readings.

|  | Description | Sample 1 | Sample 2 | Sample 3 |
| --- | --- | --- | --- | --- |
| 1 | Volume of sample sand (V2) |  |  |  |
| 2 | Volume of silt layer (V1) |  |  |  |
| 3 | Percentage of silt (calculate this as follows: (V1/V2) x 100 = % of silt) |  |  |  |
|  | Average silt content |  |  |  |

- A slump test on a concrete mix using the following link and equipment supplied.

https://www.youtube.com/watch?v=yzpWGrh9j6Y

Follow you tutor’s instructions to carry out the slump test. Observe and measure the results. What did you notice? Did the concrete mix collapse, shear or slump? What did this tell you about the mix?

- Carry out a brick absorption test.

  - Dry out some bricks in an oven

  - Weigh each brick and number it

  - Half submerge each brick in water for one hour

  - Weigh each brick

  - Calculate the % of water absorbed