# 2. Construction science principles

# Multiple choice questions (tutor)

Mortar is best mixed by?

- Weight

- Bags

- Volume

- Silo

When a beam deflects under a load the force in the top of the beam is called?

- Shear

- Bending

- Tension

- Compression

Which is the test for workability of concrete?

- Compaction

- Slump

- Silt

- Cube

In order for rust to occur in a ferrous metal, water, iron and what must be present?

- Air

- Copper

- Oxygen

- Carbon

The definition of work is?

- Gravity x distance

- Conversion to power

- Rate of energy

- Energy transferred by force

A type of renewable fuel used to produce energy is?

- Gas

- Wind

- Nuclear

- Coal

Domestic voltage use is?

- 12v

- 110v

- 230v

- 400v

The two components in the calculation of stress are force and?

- Area

- Volume

- Weight

- Mass

What part of the building regulations govern the structure?

- B

- A

- R

- E

A U-value is used to measure what from a building element?

- Heat gain

- Long-term maintenance

- Economic stability

- Heat loss

The orientation of a building maximises?

- Sunshine

- Shading

- Daylight

- Cooling

Sound is measured in units of?

- Newtons

- Decibels

- Pressure

- Volume

The approved document that deals with the reduction of sound is?

- E

- F

- G

- J

Low energy artificial lighting is called?

- Incandescent

- Halogen

- Fluorescent

- LED

Condensation can be removed from a home by?

- Additional heating

- Closing all windows

- Extract ventilation

- Management of Data Regulations

A hole for services should be drilled through a beam at the?

- Beam top

- Neutral axis

- Ends

- Middle

A bending moment is force x ?

- Gravity

- Reaction

- Width

- Distance

Reaction forces have to equal what force?

- Opposite

- Shear

- Gravitational

- Strain

The average time for curing concrete is how many days?

- 5

- 6

- 7

- 8

A test to determine how much water a brick can hold is know as a?

- Corrosion test

- Gauging test

- Compressive test

- Absorption test