5. Sustainability principles

# Worksheet 17: Renewable energy evaluation report (learner)

- A client has appointed a design and build contractor to build a new four-bedroom detached house. The house is to be located on a site surrounded by existing housing built in the 1930s with limited garden space. Planning conditions for the new house state that it should be sustainable and in keeping with the architecture of the local built environment.

The client has not specified an exact budget or timescale for the house to be built but has indicated that they are looking for environmental sustainability technology to be incorporated into the build. The build has limited garden space and neighbouring properties nearby.

The architect has put forward the following three options.

- Solar PV

- Micro wind

- Ground source heat pump

Evaluate these options to produce a justified recommendation for the client.

Your report should include the potential advantages and disadvantages of each system, and any considerations for the installation of each system. It should conclude by making a justified recommendation to the client.