8. Electrical and electronic principles

# Worksheet 11 Delving deeper (tutor)

Use your existing knowledge and conduct research as required to answer the following questions.

- How would you smooth the output of a full wave bridge rectifier?

Connect a capacitor across the output of the circuit.

- What type of diode can give off light?

Light emitting diode.

- What is a Zener diode?

A special type of diode used for voltage regulation. It is used in the reverse direction and has a very specific breakdown voltage which can keep voltages across it stable.

- Briefly explain how phototransistors work, and give an application.

A phototransistor’s base is designed to react to light. When light shines on the base the current will flow between collector and emitter. Used for light sensitive switches.

- How do you test a transistor with a multimeter?

npn: use a multimeter on the diode testing position. Connect +ve lead (red) to the base and the -ve to the collector. You should read a volt drop of approx. 0.7V. Now connect the -ve to the emitter and again observe a volt drop of approx. 0.7V. Reverse the leads in both cases and you should read O/L.

pnp: put the -ve lead on the base and the +ve on the collector. You should read approx. 0.7V. Put the +ve lead on the emitter and again you should read approx. 0.7V. Reverse the leads and you should see O/L.