8. Electrical and electronic principles

# Worksheet 12: Protection systems, waveforms and signals (learner)

- A short circuit fault has occurred down the lengths of three cables with resistances of 0.05Ω, 0.075Ω and 0.025Ω. The supply is 440V. Estimate the prospective fault current.

- 1800A

- 2933A

- 3060A

- 2514A

- Which of the following two statements are true?

- Currents are less the closer they are to the supply fault

- Currents are less the further away they are from the supply fault

- Fault currents are independent of distance from the supply

- Fault currents are greater closer to the supply

- Why are transformers used in protection equipment? Two reasons shown are correct.

- To step up the current and voltage

- To step down the current and voltage

- To create isolation between the fault currents and the protection circuit

- To keep the fault currents low

- Identify two functions of a distance relay.

- To measure the current at different distances along a cable

- To indicate where the fault is from the supply

- To trip the circuit when power increases

- To trip the circuit when current increases and voltage reduces

- Which of the following two statements are valid?

- The higher the fault current the lower the trip time

- The lower the fault current the lower the trip time

- The lower the time the higher the trip current

- The lower the time the lower the fault current

- Which is the best description of the function of an RCD?

- To trip a circuit when current gets too high

- To trip a circuit when there is a difference between two supply lines

- To trip a circuit when there is a small difference of current between two supply lines

- To trip short circuit currents

- Why do we have a non-linear resistor in a lightning arrester?

- To reduce the current when the voltage is high

- To protect the spark gap

- The large voltage causes the resistor value to be low

- To limit current flow to earth

- Which waveform is not made up of multiple frequencies?

- Rectangular wave

- Sine wave

- Square wave

- Saw tooth wave

- Which of the following is not an analogue waveform?

- Half wave rectified DC

- Full wave rectified DC

- The output of a thermocouple

- The output of an A/D

- Identify two purposes of a P/I converter.

- To allow an analogue signal to be digital

- To convert power to current

- To allow a unit bar of pressure to be converted to a current

- To allow a pressure signal to be connected to an electrical circuit