8. Electrical and electronic principles

# Worksheet 7: DC circuits (learner)

- A capacitor of 500uF in series with a resistor of 200Ω is connected to a supply of 24V DC.

Calculate:

- The circuit time constant

- The maximum current

- The time taken for the capacitor to become fully charged.

- The time taken for the capacitor to charge to 12V

- A capacitor is connected in series to a resistor of 2kΩ with a supply voltage of 150V DC. When the capacitor is fully charged the energy stored is 4.5J. Calculate the value of the capacitor.

- A resistance of 10Ω and inductance L are connected in series. The time constant of the circuit is 5ms. It is connected across a supply of 20V.

Calculate:

- The value of inductance L

- The circuit steady state current

- The value of current after 0.52ms

- A coil has a resistance of 25Ω and the final energy stored is 1.2J. The supply voltage is 50V DC.

Calculate:

- The coil inductance

- The circuit time constant

- The time taken for the maximum steady state current

- The time taken to reach 50% of the final steady state current