11. Quality management

# Worksheet 10: Writing an SOP (tutor)

Write a step-by-step SOP for the operation of a machine of your choice. Include the

following:

- Title

Drill press machine

- List of required equipment and materials

Workpiece (steel plate), machine vice, 5mm drill bit

- Potential hazards

Flying workpiece, physical injury, trapping, entanglement, manual handling

- Skills required

Machine training

- Number of personnel needed

One operator

- Time needed to complete

30 minutes to drill three holes

- Recommended frequency for completing the task

Batches of 10

- Steps to follow

Be aware of the location of start and stop switches or buttons

Keep the drill press table free of tools and other materials

Use only properly sharpened drill bits, sockets and chucks in good condition

Do not insert a drill chuck key into the chuck until the power is shut off and the machine has come to a complete stop

Insert 5mm drill bit into the chuck and secure

Ensure chuck guard is in place

Secure machine vice to drill table

Hold steel plate on machine vice

Turn on drill and drill 5mm hole ...